##### CRN: 50295

School of Computing, Science & Engineering

SEMESTER ONE EXAMINATION

PROGRAMMES:

BEng Control and Automation

BLOCK CODES:

E/CAU/F

AUTOMATION AND ROBOTICS

Date:	2nd December 2022			                            Time: _:_ -_:_

Instructions to Candidates

Time allowed 2 hours

Answer any THREE questions.

Each question carries 20 marks

The use of approved electronic calculators is allowed

Given a system has the following transfer function:

Gs= s+1s2+1

    - Determine the poles and zeros of the open and closed loop systems shown in the figure below and sketch them on an s-plane graph.                     (10 marks)

    - Are the systems stable and why?                                                         (4 marks)

    - Predict and draw the response of the systems to a unit step input.      (6 marks)

(i)

Open loop

C(s)R(s)=Gs= s+1s2+1

Zeros:

s+1=0→s= -1  One simple zero

Poles:

s2+1=0

s2=-1→s=±1j  Two imaginary poles                               (4 marks)

Closed-loop

C(s)R(s)=G(s)1+GsH(s)=G(s)1+Gs=s+1s2+11+s+1s2+1=s+1s2+s+2

Zeros:

s+1=0→s= -1  One simple zero

Poles:

s2+s+2=0→s=12(-1±7j)  Two complex poles                  (6 marks)

(ii)

Two mirrored poles on the jw axis. Undamped unstable system.            (2 marks)

Two poles are lying on the left-hand side of the s-plane. A Stable system.      (2 marks)

(3 marks)

- marks)

        - A system has a characteristic equation

qs=s5+s4+2s3+s2+s+5=0

          - Using the Routh-Hurwitz method, determine whether the system is stable.

(5 marks)

          - If it is not stable, how many poles are in the right half-plane?        (5 marks)

  - Calculate the position of the end effector of the arm below from the joint angles given.                                                                                                    (10 marks)

θ1=30°, θ2=150°, θ3=210°,θ4=120°

(a)

Examining the first column of the Routh array, the system is unstable.

s5s4s3s2s1s01115-55        21-45        15

(5 marks)

(ii)

Two sign changes in the first column hence, two poles are in the right half-plane

(5 marks)

(b)

x1=0.6cos30=0.52m

y1=0.6sin30=0.3m

x2=0.4cos0=0.4m

y2=0.4sin0=0m

x3=0.3cos30=0.26m

y3=0.3sin30=0.15m

x4=0.6cos(-30)=0.52m

y4=0.6sin(-30)=-0.3m

The position of the end effector

xg=x1+x2+x3+x4=1.7m

yg=y1+y2+y3+y4=0.35m

(xg,yg)=(1.7m,0.15m)                                                                                           (10 marks)

The closed-loop speed control of a car is represented by the model shown in the Figure 1.

Figure 1. Closed-loop speed control of a car

Given the system has the following transfer function:

C(s)R(s)=264s2+10s+264

and the general transfer function for a second order closed-loop control system is:

- Determine the un-damped natural frequency.                                                  (4 marks)

- Determine the damping ratio.                                                                          (4 marks)

- Determine the peak time of the system.                                                           (4 marks)

- Determine the percentage maximum overshoot.                                             (4 marks)

- Determine the settling time to 2% of the steady state value.                           (4 marks)

- From the general form of the transfer function:

ωn2=264→ωn=264=16.25  rad/s                                (4marks)

- From the general form of the transfer function:

2ζωn=10

So

ζ=102264=0.31                                                            (4 marks)

(iii)

tp=πωn1-ζ2

tp=3.1416.241-0.0961=0.2s                                                  (4 marks)

(iv)

Mp=100e-  πζ1-ζ2=36%                                                   (4 marks)

(v)

Settling time to 2% is given by:

ts=4ωnζ=416.25×0.31=0.79s                                                (4 marks)

An object is grasped and lifted by a manipulator as shown in the Figure 2. The manipulator is made of one revolute joint J1, one linear joint J2 and a parallel jaw gripper. The total mass of the joint J2 is 8 kg including its gripper, arm and grasped object. Calculate the time takes to move the object 0.5 m along the direction shown in the figure 2.

Assuming:

The encoder of the J1 is showing -45°.

The maximum velocity of the actuator J2 is 0.2 m/s.

The maximum Force of the actuator J2 is 400 N.

Maximum braking force (300 N)

Friction force is (2 N)

Damping force (5N)

Figure 2.

(20 marks)

Acceleration mode: The brake is off so:

Fa=400-5-2+8×9.8×cos45=448.4 N

So, acceleration can be calculated as

a=448.48=56 ms2

and

ta=Vfa=0.0035 s

da=Vit+12at2=0.00034m=0.34mm

(5 marks)

Braking mode

Fb=300+2+5-8×9.8×cos45=251.56N

ad=251.568=-31.45 m/s2

tb=Via=0.231.45=0.006s

db=Vit+12at2=0.2×0.006+0.5×-31.45×0.0062=0.00063m=0.63mm

(5 marks)

Distance cruising

dc=0.5-0.00034-0.00063=0.499m

(5 marks)

The time in the cruising phase can be calculated as

tc=dcV=2.495 s

t=ta+tb+tc

(5 marks)