AUTOMATION AND ROBOTIC TUTORIAL 1

- Look at the following control functions. Are they linear/non-linear, time invariant/time variant?

yt+2u2t+7ut+4t

yt+4ut+7c+4

yt+2tu3t+3ut

- Feedback Calculation

Using the standard feedback arrangement from the lecture:

Calculate the required values to produce a stable voltage gain of 100 using a high-gain active amplifier with a gain of 100,000.

Determine the effect on the overall gain of the circuit if the gain of the active amplifier varies from 100,000 to 200,000.