AUTOMATION AND ROBOTIC TUTORIAL 2 EXTRAS

- Derive the transfer function of a permanent magnet DC generator if the                            torque applied to the rotor is τin and the output from the coil is the current, i.  Assume the rotor is frictionless and has a radius a as shown below.

The free body diagram and schematic for this will be:

| Mechanical Free Body Diagram |  | Electrical Schematic |
| --- | --- | --- |

There are there torques involved.

τin is the external torque applied to the generator.

τe is the torque from the induces current in the coil that resists the input torque.

Jω is the torque from the moment of inertia J of the rotor.

α is a term that accounts for the magnetic flux β passing through a coil of rectangular dimensions 2aℓ and of n turns

em is the emf generated by coil rotating at an angular velocity ω

We want to relate the Torque in to the current out so we are looking for the transfer function:

G=I(s)Τin(s)

We have the equations:

αi+ τin+  Jω =0

And

αω+  iR=0

The time varying quantities in these equations are i, ω and τin

So we need to make Laplace substitutions for each of these in the equations.

αI(s)+ Τin(s)+  JsΩ(s)=0

And

αΩ(s)+  I(s)R=0

(Ω is the capital form of ω.)

We want to keep I(s) and Τin(s) but eliminate Ω(s)

To do that make Ω(s) the subject of one of the equations.

αI(s)+ Τin(s)+  JsΩ(s)=0

Subtract JsΩs from each side to give:

αIs+ Τins=- JsΩs

Then divide by – Js to leave:

-αIs+ ΤinsJs=Ωs

Now substitute into the other equation

α-αIs+ ΤinsJs+  IsR= -α2Is+ αΤinsJs+  IsR=0

Multiply the equation by -Js

α2Is+ αΤins-  IsRJs=0

Move term in Τins by subtracting from each side of the equation

α2Is-  IsRJs=- αΤins

Extract Is as a common term

Is(α2-  RJs)=- αΤins

Divide by Τins  and (α2-  RJs) to get your transfer function.

IsΤins=- α(α2-  RJs)

- Consider the following transfer functions.

Determine

  - Whether the transfer function is proper or improper

  - Poles of the system

  - zeros of the system

  - Order of the system

- Gs=s2+2s+13s2+s-4

Function is improper as denominator is not a higher order than numerator

As it is second order the number of poles and zeros must be 2

For numerator s2+2s +1=0 so applying  x=-b±b2-4ac2a  there are two real zeros of both at -1

For denominator  3s2+s-4=0 so applying  x=-b±b2-4ac2a  there are two real polls of 1

X

-1.33

s-plane

X

1

0

jω

and -1.33

0

-1

- Gs=2s+3s2+2s+3

Function is proper as denominator has higher order than numerator

The numerator is first order so the must be 1 zero

The numerator is second order the number of poles must be 2

For numerator 2s+3=0 or s=-3/2 so a real zero at -1.5

For Denominator  s2+2s+3=0 so applying  x=-b±b2-4ac2a  there are two imaginary roots of -1, 1.41i

X

-1.41

s-plane

0

-1.5

X

1.41

jω

and -1, -1.41i

-1

- Gs=4s2-2s+1

Function is proper as denominator is not a higher order than numerator

There are no zeros

As denominator is second order the number of poles must be 2

For Denominator  s2-2s+1=0 so applying  x=-b±b2-4ac2a  there are two real roots both at 1

X

s-plane

X

1

jω