AUTOMATION AND ROBOTIC TUTORIAL 2 SOLUTIONS

- A common actuator in control systems is the DC motor. It directly provides rotary motion and, coupled with wheels or drums and cables, can provide translational motion. The electric equivalent circuit of the armature and the free-body diagram of the rotor are shown in the following figure.

We will assume that the input of the system is the voltage source () applied to the motor's armature, while the output is the rotational speed of the shaft . The rotor and shaft are assumed to be rigid. We further assume a viscous friction model, that is, the friction torque is proportional to shaft angular velocity.

In general, the torque generated by a DC motor is proportional to the armature current and the strength of the magnetic field. In this example we will assume that the magnetic field is constant and, therefore, that the motor torque is proportional to only the armature current  by a constant factor  as shown in the equation below. This is referred to as an armature-controlled motor.

The back emf, , is proportional to the angular velocity of the shaft by a constant factor .

The motor torque and back emf constants are equal, that is, ; therefore, we will use  to represent both the motor torque constant and the back emf constant.

From the figure above, we can derive the following governing equations based on Newton's 2nd law and Kirchhoff's voltage law. (These equations would be given to you as the starting point for any question.)

Find the transfer function to represent the gain in rotational speed as a function of voltage P(s) where:

You can assume all starting conditions are zero.

So we need to create an equation that has Θ(s)Vs on one side and just constants on the other side to allow us to create a transfer function. This means I(s) will be a problem so we will try and eliminate it at some point.

Start by applying the laplace transform to each of our differential equations. This requires looking at each differential term in the equation and replacing it with its Laplace equivalent. Taking

first. The θ is the same as d2θdt2 for which our laplace substitution is Θs2 so the first term Jθ is replaced by JΘs2

The θ is the same as dθdt for which our laplace substitution is Θs so the second term bθ is replaced by bΘs

The Ki has no differential and so is unchanged.

After the laplace transform the variables θ and i are now functions of s, so the formula becomes: (Lower case letters are normally replaced by the upper case letter to indicate the change to a function of s)

Js2Θs+bsΘs=KI(s)

We can remove sΘ(s) as a common term by splitting the s2 into s.s. Common terms are useful to create things we can divide or multiply through later to move them around the equation. Here the thinking is that we want  Θ(s) in the final transfer function Θ(s)Vs and as sΘs= Θ(s) extracting it now will help us get that  Θ(s) term later. So:

JssΘs+b  sΘs=KI(s)

Which is :

sΘs (Js+b)=KI(s)

(Multiply the sΘs back through to check you get the original equation if you want to check your working)

Looking now at the second equation;

The didt can be replaced using a laplace substitution by Is and the θ which is the same as dθdt for which our laplace substitution is Θs. So the equation becomes:

LIss+RIs=Vs-KΘss

Taking out I(s) as a common term gives;

Now one of the things we want to do is get rid of I(s). The standard way of doing this if we have two equations that have I(s) in them is to make I(s) the subject of one equation and then substitute the result of that into any position in the second equation where I(s) occurs. So going back to our fist equation above:

sΘs (Js+b)=KI(s)

Divide the equation through by K to get it in terms of I(s) so:

sΘs(Js+b)K=I(s)

Now put this equivalence for I(s) into our other equation

To get:

Ls+RsΘ(s)(Js+b)K=Ls+RsΘ(s)(Js+b)K=Vs-KsΘ(s)

We want to get Ps=Θ(s)V(s)

So we need to create an Θ(s) term

We know sΘs= Θ(s) from above so we can make the substitution for both occurences of sΘs in the equation giving:

Ls+R(Js+b)Θ(s)K=Vs-KΘ(s)

By adding KΘs to both sides of the equation we can eliminate it from one side  and  give us:

Ls+R(Js+b)Θ(s)K+KΘ(s)=Vs

Remember we are trying to get the transfer function

Ps=Θ(s)V(s)

and so are aiming to generate this term by having all  Θ(s) terms on one side of the equation and Vs terms on the other

We can treat this equation just like a fraction where you would cross multiply to get a common denominator:

Ls+R(Js+b)Θ(s)K+KΘs=Ls+R(Js+b)Θ(s)K+KΘ(s)1=Vs

Ls+R(Js+b)Θ(s)K+K2Θ(s)K=Vs

Ls+R(Js+b)Θ(s)+ K2Θ(s)K=Vs

Pulling out  Θ(s) again as a common factor gives:

(Ls+RJs+b+K2)Θ(s)K=Vs

Now we can divide by  Θ(s) to get

(Ls+RJs+b+K2)K=VsΘ(s)

Almost what we want but the wrong way up

So we can just invert it to arrive at the final transfer function

- Consider the following transfer functions.

Determine

  - Whether the transfer function is proper or improper

  - Poles of the system

  - zeros of the system

  - Order of the system

Function is proper as denominator has higher order than numerator

As it is second order the number of poles must be 2

For numerator s+3=0 so s=-3 or a zero at -3

For Denominator  s2+2s=0 so applying  x=-b±b2-4ac2a  there are two roots of -2

X

-2

s-plane

X

0

0

-3

jω

and 0

Function is proper as denominator has higher order than numerator

As it is second order the number of poles must be 2

For numerator s=0 or a zero at 0

For Denominator  s2+5s+6=0 so applying  x=-b±b2-4ac2a  there are two roots of -2

X

-2

s-plane

0

0

X

-3

jω

and -3

Function is improper as denominator is not a higher order than numerator

As it is second order the number of poles must be 2

For numerator s2+6s+9=0 so applying  x=-b±b2-4ac2a  there is one 0 at -3

For Denominator  s2-10s=0 so applying  x=-b±b2-4ac2a  there are two roots of 0

X

10

s-plane

X

0

0

-3

jω

and 10

Function is improper as denominator is not a higher order than numerator

As it is second order the number of poles must be 2

For numerator s2+s=0 so applying  x=-b±b2-4ac2a  there is two 0 at -1

For Denominator  s2+s +4=0 so applying  x=-b±b2-4ac2a

We get:   x=-1±12-4.1.42.1= x=-1±-152

We are not allowed a negative value as a square root we can extract   -1=i and make it an imaginary number

This gives x=-1±i152

As we cannot combine real and imaginary numbers this must be split into two parts

x=-12±i152

there are two complex roots of -0.5, + 1.93i and  -0.5, -1.93i

X

s-plane

0

0

-1

jω

-2

+2

X