AUTOMATION AND ROBOTIC TUTORIAL EXTRAS 6

1)

Calculate the position of the end effector of the linear arm below from the joint angles given.

y3

Θ5

0.6m

y2

x3

0.8m

1m

Θ3

x2

Θ2

y1

Θ4

Θ1

x1

(0,0)

Θ1=45o, Θ2=140o, Θ3=230o,

Divide problem up into right angle triangles.

x1=1 cos 45=0.707m, y1=1 sin 45=0.707m

so θ4=140-90-45=5 (the 45 comes from 180-90-ϴ1 in the first triangle)

x2=0.8 cos 5=0.797m, y2=0.8 sin 5=0.07m

θ5=230-90-85=55 (the 85 comes from 180-90-ϴ4 in the second triangle)

x3=0.6 cos 55=0.344m, y3=0.6 sin 55=0.491m

add up x’s and y’s for final co-ordinate

(1.848, 1.268)

2)

A robot arm is required to move to 3 different co-ordinates above a sheet of metal to perform a drilling operation. Produce an inverse kinematic solution to calculate the angular positions (θ1 &  θ2 ) of joints 1 & 2 required to position the arm at the three Cartesian coordinates shown.

1m

1.5m

(1.6,1)

(1.2,0.8)

(1,0.5)

(0,0)

Θ1

Θ2

First construct simple kinamatic equations by trigometiric construction

1.5m (L1)

1m(L2)

(0,0)

(X,Y)

Θ1

Θ2

B

K2

K1

First us the X and Y final coordinate of the tool to construct a right angled triangle where the hypotenuse is B. From this we can get:

B2 = X2+Y2

K1=tan-1(Y/X)

Now we need to find K2. To do this we will need the cosine rule to derive a formula that relates the angle  K2 to the lengths of the sides. The cosine rule is:

So if K2 is the angle we are looking for we can map the triangle fromed by sides L1, L2 and B to the triangle above so :

A=K2, b=L1, a = L2, c = B

Giving

L22 = L12 + B2+ - 2.L1.B.cos K2.

Rearranging

2. L1.B.cos K2 = L12+ B2- L22

cos K2 = (L12+ B2- L22)/ 2.B.L1

K2=cos-1((L12+ B2-L22)/2BL1)

Θ1= K1+ K2

Now Θ2 is the angle we are looking for we can map the triangle fromed by sides L1, L2 and B to the triangle above again but this time :

A= Θ2, b=L1, c=L2 and a=B

Doing the same rearrangement process

Θ 2=cos-1((L12+L22-B2)/2L1 L2)

For (1.6,1)

These formula allow us to calculate B=1.89, K1=32deg, K2=31.79deg

Hence Θ1=63.79deg, Θ2=96.16deg

For (1.2,0.8)

These allow us to calculate B=1.44, K1=26.56deg, K2=41.8deg

Hence Θ1=73.39deg, Θ2=66.91deg

For (1,0.5)

These allow us to calculate B=1.11, K1=19.65deg, K2=56.2deg

Hence Θ1=68.37deg, Θ2=47.72deg